Faculty Of Engineering

Advance Computer Network

Laboratory Manual

NAME: …………………………………………………………………………………………………………

ENROLLMENT NUMBER: ………………………………………………………………………….

SUBJECT NAME: ……………………………………………………………………………………….

SUBJECT CODE: …………………………………  BRANCH: ……………………………………

SEMESTER:  ………………………          ACADEMIC YEAR: ………………………………………

ISBN Number -

Swaminarayan university Publication

Certificate

This is to certify that Mr. / Ms. / Mrs. …………………………….……………………………… having enrollment number ……………………………………… of ….............. Semester ME / BE / DIPLOMA in ………….……….……….………..……….. has satisfactorily completed his term working (Subject name ……………………………………......) (subject code: ………….) during academic year ………….... at Faculty of Engineering, Swaminarayan University.

Place: ……………….

Date: ….....................

Sonal Prajapati                                                                                          Dr.Bhavana Soni

SUBJECT IN-CHARGE		               HOD 				DEAN

List of Practical

| Sr. No. | Title | Page No. | Date | Sign |
| --- | --- | --- | --- | --- |
| 1 | a) Rewrite the following IP addresses in binary notation 192.168.12.79 156.152.187.189 172.16.12.11 10.159.187.198 b) Rewrite the following IP addresses in dotted decimal notation 11000000.10101000.00111000.00001100 10101001.00001100.00001011.01001111 00001010.10111011.00001100.01111011 01111101.10111011.11100000.11111111 10101001.00001100.00001011.01001111 00001010.10111011.00001100.01111011 01111101.10111011.11100000.11111111 c) Consider the following IP addresses 214.229.206.83/28 153.120.147.39/26 115.173.104.1/18 70.173.166.71/1 Find the following for each above IP address 1. Network Address 3. First Host Address Last Host Address 4. Broadcast Address 5. Next Subnet Address d) An organization is granted block 212.18.190.0/24. The administrator wants to create 32 subnets. 1. Find the subnet mask. 2. Find the number of addresses in each subnet. 3. Find the first and last address in subnet 1 2. Find the first and last address in subnet 32. | 7 |  |  |
| 2 | What is the header size? Are there any options in the packet? What is the size of data? Is the packet fragmented? How many more routers can the packet travel to? What it the protocol number of the payload being carried by the packet? | 11 |  |  |
| 3 | Capture and study ICMPv4 packets generated by Other utility programs such as ping and traceroute using relevant software | 12 |  |  |
| 4 | Create a small IPv4 static routing network using relevant software. | 14 |  |  |
| 5 | Create a small IPv6 network using any relevant software | 16 |  |  |
| 6 | Configure RIP routing protocol using relevant software | 18 |  |  |
| 7 | Configure OSPF routing protocol using relevant software | 19 |  |  |
| 8 | Configure BGP routing protocol using relevant software. a) The following is a dump (contents) of a UDP header in hexadecimal format. 0045DF0000580000 | 21 |  |  |
| 9 | b.What is the destination port number? c. What is the total length of the user datagram? d.What is the length of the data? e. Is the packet directed from a client to a server or vice versa? f. What is the application-layer protocol? g. Has the sender calculated a checksum for this packet? The following is part of a TCP header dump (contents) in hexadecimal format. E293 0017 00000001 00000000 5002 07FF. a. What is the source port number? b.What is the destination port number? c. What is the sequence number? d.What is the acknowledgment number? e. What is the length of the header? f. What is the type of the segment? g.What is the window size? c) The following is a dump of an SCTP general header in hexadecimal format. 04320017 00000001 00000000 a. What is the source port number? b.What is the destination port number? c. What is the value of the verification tag? d)What is the value of the checksum? | 24 |  |  |
| 10 | Capture and Study TCP and UDP Packets using relevant software. | 27 |  |  |
| 11 | Configure Dynamic Host Configuration Protocol using relevant software | 30 |  |  |
| 12 | a) Configure Domain Name Server (DNS) using relevant software. b) Configure Web Server using relevant software. | 32 |  |  |
| 13 | Configure File Transfer Protocol (FTP) using relevant software. | 34 |  |  |
| 14 | Configure Mail Server Using relevant software. | 35 |  |  |

PRACTICAL-1

AIM: a) Rewrite the following IP addresses in binary notation

192.168.12.79

156.152.187.189

172.16.12.11

10.159.187.198

b) Rewrite the following IP addresses in dotted decimal notation

11000000.10101000.00111000.00001100

10101001.00001100.00001011.01001111

00001010.10111011.00001100.01111011

01111101.10111011.11100000.11111111

10101001.00001100.00001011.01001111

00001010.10111011.00001100.01111011

01111101.10111011.11100000.11111111

c) Consider the following IP addresses

214.229.206.83/28

153.120.147.39/26

115.173.104.1/18

70.173.166.71/1

Find the following for each above IP address

1. Network Address

3. First Host Address Last Host Address

4. Broadcast Address

5. Next Subnet Address

d) An organization is granted block 212.18.190.0/24. The administrator

wants to create 32 subnets.

1. Find the subnet mask.

2. Find the number of addresses in each subnet.

3. Find the first and last address in subnet 1

2. Find the first and last address in subnet 32.

# a) Rewrite these IP addresses in binary (each octet = 8 bits)

- 192.168.12.79
192 = 11000000
168 = 10101000
12 = 00001100
79 = 01001111
→ 11000000.10101000.00001100.01001111

- 156.152.187.189
156 = 10011100
152 = 10011000
187 = 10111011
189 = 10111101
→ 10011100.10011000.10111011.10111101

- 172.16.12.11
172 = 10101100
16 = 00010000
12 = 00001100
11 = 00001011
→ 10101100.00010000.00001100.00001011

- 10.159.187.198
10 = 00001010
159 = 10011111
187 = 10111011
198 = 11000110
→ 00001010.10011111.10111011.11000110

# b) Rewrite these binary addresses in dotted-decimal

I’ll convert each 8-bit group to decimal.

- 11000000.10101000.00111000.00001100
11000000 = 192, 10101000 = 168, 00111000 = 56, 00001100 = 12
→ 192.168.56.12

- 10101001.00001100.00001011.01001111
10101001 = 169, 00001100 = 12, 00001011 = 11, 01001111 = 79
→ 169.12.11.79

- 00001010.10111011.00001100.01111011
00001010 = 10, 10111011 = 187, 00001100 = 12, 01111011 = 123
→ 10.187.12.123

- 01111101.10111011.11100000.11111111
01111101 = 125, 10111011 = 187, 11100000 = 224, 11111111 = 255
→ 125.187.224.255

- (repeat) 10101001.00001100.00001011.01001111 → 169.12.11.79

- (repeat) 00001010.10111011.00001100.01111011 → 10.187.12.123

- (repeat) 01111101.10111011.11100000.11111111 → 125.187.224.255

# c) For each IP with prefix — find Network Address, First host, Last host, Broadcast, Next subnet

General notes:

- Network address = clear host bits to 0 according to prefix.

- Broadcast = set host bits to 1.

- First host = network + 1 (unless /32).

- Last host = broadcast − 1.

- Next subnet address = network + block size.

### 1) 214.229.206.83/28

- /28 → mask = 255.255.255.240. Host bits = 4 → block size = 2⁴ = 16 in last octet.

- Last octet 83 → integer div: 83 // 16 = 5 remainder 3 → network last-octet = 5 * 16 = 80.
Network address: 214.229.206.80
First host: 214.229.206.81
Broadcast: network + 15 = 214.229.206.95
Last host: 214.229.206.94
Next subnet address: 214.229.206.96

### 2) 153.120.147.39/26

- /26 → mask = 255.255.255.192. Host bits = 6 → block size = 64 in last octet.

- Last octet 39 → 39 // 64 = 0 → network last octet = 0.
Network address: 153.120.147.0
First host: 153.120.147.1
Broadcast: 153.120.147.63
Last host: 153.120.147.62
Next subnet address: 153.120.147.64

### 3) 115.173.104.1/18

- 18 → mask = 255.255.192.0. (16 bits + 2 bits of third octet). Host bits in third+fourth = 14. Third-octet block size = 256 - 192 = 64.

- Third octet 104 → 104 // 64 = 1 remainder 40 → network third octet = 1 * 64 = 64.
Network address: 115.173.64.0
First host: 115.173.64.1
Broadcast: third octet = 64 + 63 = 127, last octet 255 → 115.173.127.255
Last host: 115.173.127.254
Next subnet address: 115.173.128.0

### 4) 70.173.166.71/1

- /1 → mask = 128.0.0.0. Only the highest bit of first octet is network. Blocks are size 128 in first octet.

- First octet 70: 70 // 128 = 0 → network first octet = 0. So this IP falls in the 0/1 half.
Network address: 0.0.0.0 (i.e. the /1 network covering 0.0.0.0–127.255.255.255)
First host: 0.0.0.1
Broadcast: 127.255.255.255
Last host: 127.255.255.254
Next subnet address: 128.0.0.0

# d) Organization has block 212.18.190.0/24. Admin wants 32 subnets.

- Subnet mask: To get 32 subnets from a /24 you need 32 = 2^5 → borrow 5 bits from host portion. New prefix = 24 + 5 = /29.
→ Subnet mask = 255.255.255.248.

- Number of addresses per subnet: host bits = 32 - 29 = 3 → 2^3 = 8 addresses per subnet (includes network and broadcast).

- Subnet 1 (we take subnet numbering starting at 1 = the first subnet at base address)

  - Subnet size = 8 addresses. Subnet 1 network = 212.18.190.0/29. Range .0 to .7.

  - Network (subnet 1) = 212.18.190.0

  - Broadcast (subnet 1) = 212.18.190.7

  - Usable hosts (subnet 1) = 212.18.190.1 ... 212.18.190.6
(So first usable = .1, last usable = .6.)

- Subnet 32

  - Subnet N network = base + (N−1)*8. For N=32 → offset = 31 * 8 = 248.

  - Network (subnet 32) = 212.18.190.248/29

  - Broadcast (subnet 32) = 212.18.190.255

  - Usable hosts (subnet 32) = 212.18.190.249 ... 212.18.190.254
(first usable .249, last usable .254).

PRACTICAL-2

AIM: What is the header size?

Are there any options in the packet?

What is the size of data?

Is the packet fragmented?

How many more routers can the packet travel to?

What it the protocol number of the payload being carried by the packet?

- Header size: IHL = 5 → 5 × 4 = 20 bytes (160 bits).

- Are there any options in the packet? No.

- IHL = 5 is the minimum (no options).

- Size of data (IP payload): Total Length − Header Length = 84 − 20 = 64 bytes.

- (Total Length = 0x0054 = 84 decimal; payload = the bytes after the 20-byte IP header — includes transport header + application data.)

- Is the packet fragmented? No.

- Flags+Fragment offset = 0x0000 → fragment offset = 0 and MF (more fragments) = 0 → not fragmented.

- How many more routers can the packet travel to? Up to 32 more hops.

- TTL = 0x20 = 32; each router decrements TTL by 1; when TTL reaches 0 the packet is dropped.

- Protocol number of the payload: 0x06 → 6 = TCP.

PRACTICAL-3

AIM: Capture and study ICMPv4 packets generated by Other utility programs such as ping and traceroute using relevant software.

## 1. Tools you will need

- Wireshark (GUI packet sniffer, cross-platform)

- Or tcpdump (CLI-based, Linux/Unix)

- Utility programs that generate ICMP:

  - ping

  - traceroute (Linux/macOS) or tracert (Windows)

## 2. Capturing ICMP packets with ping

- Start Wireshark and select the active network interface.

- Apply a capture filter (optional) → icmp to capture only ICMP packets.

- Open a terminal/command prompt and run:

  - Windows: ping www.google.com

  - Linux/macOS: ping -c 4 www.google.com (sends 4 packets).

- In Wireshark, you will see:

  - ICMP Echo Request (Type 8, Code 0) from your host to the destination.

  - ICMP Echo Reply (Type 0, Code 0) from the destination back to you.

Fields to study in Wireshark (ICMP header):

- Type & Code (8/0 for request, 0/0 for reply).

- Identifier (to match request/reply).

- Sequence number.

- Checksum.

## 3. Capturing ICMP packets with traceroute / tracert

Traceroute works differently depending on OS:

- Linux/macOS traceroute:

  - Sends UDP packets with increasing TTL values.

  - Each router along the path replies with ICMP Time Exceeded (Type 11, Code 0).

- Windows tracert:

  - Uses ICMP Echo Requests with increasing TTL values.

  - Intermediate routers reply with ICMP Time Exceeded until destination replies with Echo Reply.

How to capture in Wireshark:

- Apply capture filter: icmp.

- Run:

  - Windows: tracert www.google.com

  - Linux/macOS: traceroute www.google.com

- Observe:

  - Each hop sends back ICMP Time Exceeded messages.

  - The final destination sends an ICMP Echo Reply (Windows) or ICMP Port Unreachable (Linux/macOS, since traceroute uses UDP to an unused port).

## 4. Analysis checklist in Wireshark

When you study captured ICMPv4 packets, check:

- Type and Code field (Echo request/reply, Time Exceeded, Destination Unreachable).

- Checksum validity (Wireshark shows if correct).

- Sequence numbers (to match requests and replies).

- TTL changes in the IP header (each router decrements TTL).

- Round Trip Time (RTT) in ping replies, visible in Wireshark or command output.

PRACTICAL-4

AIM: Create a small IPv4 static routing network using relevant software.

## 1. Network Topology

We’ll make a 3-router static routing network with three LANs:

LAN1 (PC1: 192.168.1.0/24) --- R1 --- (10.0.0.0/30) --- R2 --- (10.0.0.4/30) --- R3 --- LAN2 (PC2: 192.168.2.0/24)

- R1–R2 link: 10.0.0.0/30 (point-to-point)

- R2–R3 link: 10.0.0.4/30 (point-to-point)

- LAN1 (behind R1): 192.168.1.0/24

- LAN2 (behind R3): 192.168.2.0/24

## 2. IP Addressing Plan

- R1:

  - G0/0: 192.168.1.1

  - S0/0/0: 10.0.0.1

- R2:

  - S0/0/0: 10.0.0.2

  - S0/0/1: 10.0.0.5

- R3:

  - S0/0/0: 10.0.0.6

  - G0/0: 192.168.2.1

- PC1: 192.168.1.10 (gateway 192.168.1.1)

- PC2: 192.168.2.10 (gateway 192.168.2.1)

## 3. Static Routing Configuration (Cisco IOS)

On R1:

Router> enable

Router# configure terminal

Router(config)# interface g0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config)# interface s0/0/0

Router(config-if)# ip address 10.0.0.1 255.255.255.252

Router(config-if)# no shutdown

# Static route to LAN2 via R2

Router(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.2

On R2:

Router> enable

Router# configure terminal

Router(config)# interface s0/0/0

Router(config-if)# ip address 10.0.0.2 255.255.255.252

Router(config-if)# no shutdown

Router(config)# interface s0/0/1

Router(config-if)# ip address 10.0.0.5 255.255.255.252

Router(config-if)# no shutdown

# Route to LAN1 via R1

Router(config)# ip route 192.168.1.0 255.255.255.0 10.0.0.1

# Route to LAN2 via R3

Router(config)# ip route 192.168.2.0 255.255.255.0 10.0.0.6

On R3:

Router> enable

Router# configure terminal

Router(config)# interface g0/0

Router(config-if)# ip address 192.168.2.1 255.255.255.0

Router(config-if)# no shutdown

Router(config)# interface s0/0/0

Router(config-if)# ip address 10.0.0.6 255.255.255.252

Router(config-if)# no shutdown

# Static route to LAN1 via R2

Router(config)# ip route 192.168.1.0 255.255.255.0 10.0.0.5

## 4. Verification

- From PC1 (192.168.1.10), ping PC2 (192.168.2.10).

- Use show ip route on each router to confirm static routes are installed.

- Use tracert (Windows) or traceroute (Linux) between PCs to see the path through R1 → R2 → R3.

PRACTICAL-5

AIM: Create a small IPv6 network using any relevant software.

Perfect 👍 — let’s build a small IPv6 network with static routing, similar to what we did for IPv4. I’ll explain the topology, addressing plan, and configuration. You can set this up in Cisco Packet Tracer, GNS3, or even Linux VMs with ip -6 commands.

## 1. Topology

## We’ll create a simple 3-router IPv6 network with two LANs:

LAN1 (PC1) --- R1 --- R2 --- R3 --- LAN2 (PC2)

## 2. IPv6 Addressing Plan

We’ll use Global Unicast Addresses (GUA) from the documentation block 2001:db8::/32.

- LAN1 (behind R1): 2001:db8:1::/64

  - R1 G0/0 → 2001:db8:1::1/64

  - PC1 → 2001:db8:1::10/64 (gateway 2001:db8:1::1)

- R1–R2 link: 2001:db8:12::/64

  - R1 S0/0/0 → 2001:db8:12::1/64

  - R2 S0/0/0 → 2001:db8:12::2/64

- R2–R3 link: 2001:db8:23::/64

  - R2 S0/0/1 → 2001:db8:23::1/64

  - R3 S0/0/0 → 2001:db8:23::2/64

- LAN2 (behind R3): 2001:db8:2::/64

  - R3 G0/0 → 2001:db8:2::1/64

  - PC2 → 2001:db8:2::10/64 (gateway 2001:db8:2::1)

## 3. Cisco IOS Configuration

### On R1

enable

conf t

ipv6 unicast-routing   # enable IPv6 routing

interface g0/0

ipv6 address 2001:db8:1::1/64

no shutdown

interface s0/0/0

ipv6 address 2001:db8:12::1/64

no shutdown

# Static route to LAN2 via R2

ipv6 route 2001:db8:2::/64 2001:db8:12::2

### On R2

### enable

conf t

ipv6 unicast-routing

interface s0/0/0

ipv6 address 2001:db8:12::2/64

no shutdown

interface s 0/0/1

ipv6 address 2001:db8:23::1/64

no shutdown

# Route to LAN1 via R1

ipv6 route 2001:db8:1::/64 2001:db8:12::1

# Route to LAN2 via R3

ipv6 route 2001:db8:2::/64 2001:db8:23::2

### On R3

enable

conf t

ipv6 unicast-routing

interface g0/0

ipv6 address 2001:db8:2::1/64

no shutdown

interface s0/0/0

ipv6 address 2001:db8:23::2/64

no shutdown

# Static route to LAN1 via R2

ipv6 route 2001:db8:1::/64 2001:db8:23::1

## 4. End Devices

- PC1: IPv6 = 2001:db8:1::10/64, Gateway = 2001:db8:1::1

- PC2: IPv6 = 2001:db8:2::10/64, Gateway = 2001:db8:2::1

## 5. Verification

- From PC1, run:

- ping 2001:db8:2::10

→ should reach PC2.

- On routers, use:

- show ipv6 route

→ to confirm static routes are installed.

- You can also use:

- traceroute ipv6 2001:db8:2::10

→ to see the path across R1 → R2 → R3.

PRACTICAL-6

AIM: Configure RIP routing protocol using relevant software

Software Used: Cisco Packet Tracer / GNS3

Steps:

- Open Cisco Packet Tracer and connect two or more routers.

- Assign IP addresses to each router interface.

- Enter configuration mode on each router:

- Router> enable

- Router# configure terminal

- Router(config)# router rip

- Router(config-router)# network 192.168.1.0

- Router(config-router)# network 10.0.0.0

- Exit and save the configuration using write or copy run start.

Verification:

- Use show ip route to check learned RIP routes.

PRACTICAL-7

AIM: Configure OSPF routing protocol using relevant software.

## 1. Topology (3 Routers + 2 LANs)

LAN1 (192.168.1.0/24) --- R1 --- R2 --- R3 --- LAN2 (192.168.3.0/24)

- R1–R2 link: 10.0.12.0/30

- R2–R3 link: 10.0.23.0/30

- LAN1 (behind R1): 192.168.1.0/24

- LAN2 (behind R3): 192.168.3.0/24

## 2. IP Addressing

- R1

  - G0/0 → 192.168.1.1/24

  - S0/0/0 → 10.0.12.1/30

- R2

  - S0/0/0 → 10.0.12.2/30

  - S0/0/1 → 10.0.23.1/30

- R3

  - S0/0/0 → 10.0.23.2/30

  - G0/0 → 192.168.3.1/24

- PC1: 192.168.1.10/24, GW=192.168.1.1

- PC2: 192.168.3.10/24, GW=192.168.3.1

## 3. OSPF Configuration (Cisco IOS)

We’ll use OSPF process ID 1 and put all routers in area 0 (backbone).

### On R1

enable

conf t

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.12.0 0.0.0.3 area 0

### On R2

enable

conf t

interface s0/0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

interface s0/0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

### On R3

enable

conf t

interface g0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

router ospf 1

router-id 3.3.3.3

network 192.168.3.0 0.0.0.255 area 0

network 10.0.23.0 0.0.0.3 area 0

## 4. Verification

- Check OSPF neighbors

show ip ospf neighbor

(You should see R1–R2 and R2–R3 adjacency.)

- Check OSPF routes

show ip route ospf

(You should see routes to remote LANs via OSPF.)

- Ping test

- From PC1 (192.168.1.10), ping PC2 (192.168.3.10).

- It should succeed once OSPF converges.

## 5. Extra: OSPF in IPv6 (OSPFv3)

- Enable IPv6 unicast routing:

- ipv6 unicast-routing

- Assign IPv6 addresses to interfaces.

- Use:

- ipv6 router ospf 1

- router-id 1.1.1.1

- interface g0/0

ipv6 ospf 1 area 0

PRACTICAL-8

AIM: Configure BGP routing protocol using relevant software.

a) The following is a dump (contents) of a UDP header in

hexadecimal format.

0045DF0000580000

## 1. Topology (3 Routers + 2 LANs)

LAN1 (192.168.1.0/24) --- R1 --- R2 --- R3 --- LAN2 (192.168.3.0/24)

- R1–R2 link: 10.0.12.0/30

- R2–R3 link: 10.0.23.0/30

- LAN1 (behind R1): 192.168.1.0/24

- LAN2 (behind R3): 192.168.3.0/24

## 2. IP Addressing

- R1

  - G0/0 → 192.168.1.1/24

  - S0/0/0 → 10.0.12.1/30

- R2

  - S0/0/0 → 10.0.12.2/30

  - S0/0/1 → 10.0.23.1/30

- R3

  - S0/0/0 → 10.0.23.2/30

  - G0/0 → 192.168.3.1/24

- PC1: 192.168.1.10/24, GW=192.168.1.1

- PC2: 192.168.3.10/24, GW=192.168.3.1

## 3. OSPF Configuration (Cisco IOS)

We’ll use OSPF process ID 1 and put all routers in area 0 (backbone).

### On R1

enable

conf t

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 10.0.12.1 255.255.255.252

no shutdown

router ospf 1

router-id 1.1.1.1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.12.0 0.0.0.3 area 0

### On R2

enable

conf t

interface s0/0/0

ip address 10.0.12.2 255.255.255.252

no shutdown

interface s0/0/1

ip address 10.0.23.1 255.255.255.252

no shutdown

router ospf 1

router-id 2.2.2.2

network 10.0.12.0 0.0.0.3 area 0

network 10.0.23.0 0.0.0.3 area 0

### On R3

enable

conf t

interface g0/0

ip address 192.168.3.1 255.255.255.0

no shutdown

interface s0/0/0

ip address 10.0.23.2 255.255.255.252

no shutdown

router ospf 1

router-id 3.3.3.3

network 192.168.3.0 0.0.0.255 area 0

network 10.0.23.0 0.0.0.3 area 0

## 4. Verification

- Check OSPF neighbors

show ip ospf neighbor

(You should see R1–R2 and R2–R3 adjacency.)

- Check OSPF routes

show ip route ospf

(You should see routes to remote LANs via OSPF.)

- Ping test

- From PC1 (192.168.1.10), ping PC2 (192.168.3.10).

- It should succeed once OSPF converges.

## 5. Extra: OSPF in IPv6 (OSPFv3)

- Enable IPv6 unicast routing:

- ipv6 unicast-routing

- Assign IPv6 addresses to interfaces.

- Use:

- ipv6 router ospf 1

- router-id 1.1.1.1

- interface g0/0

ipv6 ospf 1 area 0

PRACTICAL-9

AIM:   a. What is the destination port number?

b. What is the total length of the user datagram?

c. What is the length of the data?

d. Is the packet directed from a client to a server or vice versa?

e. What is the application-layer protocol?

f. Has the sender calculated a checksum for this packet?

g. The following is part of a TCP header dump (contents) in hexadecimal format.

E293 0017 00000001 00000000 5002 07FF.

a. What is the source port number?

b.What is the destination port number?

c. What is the sequence number?

d.What is the acknowledgment number?

e. What is the length of the header?

f. What is the type of the segment?

g.What is the window size?

c) The following is a dump of an SCTP general header in

hexadecimal format.

04320017 00000001 00000000

a. What is the source port number?

b.What is the destination port number?

c. What is the value of the verification tag?

d)What is the value of the checksum?

## 1) UDP Header Questions

(You gave generic UDP header-related questions — no dump provided, so I’ll explain with structure.)

A UDP header is 8 bytes, with the following fields:

0–15: Source Port

16–31: Destination Port

32–47: Length (UDP header + data)

48–63: Checksum

- b. Destination port number → found in bytes 2–3 of the UDP header (16-bit).

- c. Total length of the user datagram → field at bytes 4–5 (includes 8-byte UDP header + data).

- d. Length of the data → UDP length – 8.

- e. Direction (client ↔ server) → depends on whether the destination port is a well-known port (like 53 DNS, 80 HTTP, 443 HTTPS) → if destination is well-known, it’s client → server; otherwise, response from server → client.

- f. Application-layer protocol → inferred from destination port (e.g., 53=DNS, 67/68=DHCP, 69=TFTP, 161=SNMP, etc.).

- g. Has the sender calculated checksum? → If the checksum field = 0, then no (checksum not calculated); otherwise yes.

## 2) TCP Header Dump

You gave:

E293 0017 00000001 00000000 5002 07FF ..

Let’s decode carefully:

### Fields:

- Source Port (16 bits): E293 = hex → decimal 58003

- Destination Port (16 bits): 0017 = hex → decimal 23

- Sequence Number (32 bits): 00000001 = decimal 1

- Acknowledgment Number (32 bits): 00000000 = decimal 0

- Data Offset + Flags + Reserved (16 bits): 5002

  - Data Offset = high 4 bits of 0x5 → 5 × 4 = 20 bytes header

  - Flags (last 6 bits): 0x02 → SYN flag set

- Window Size (16 bits): 07FF = decimal 2047

### Answers:

a. Source port number → 58003
b. Destination port number → 23 (Telnet)
c. Sequence number → 1
d. Acknowledgment number → 0
e. Length of header → 20 bytes
f. Type of segment → SYN segment (connection request)
g. Window size → 2047

## 3) SCTP General Header Dump

You gave:

0432 0017 00000001 00000000

### SCTP Header Format:

- Source Port (16 bits)

- Destination Port (16 bits)

- Verification Tag (32 bits)

- Checksum (32 bits)

### Decode:

- Source Port = 0432 = hex → 1074

- Destination Port = 0017 = hex → 23 (Telnet again)

- Verification Tag = 00000001 → decimal 1

- Checksum = 00000000 (not computed here / placeholder)

### Answers:

a. Source port number → 1074
b. Destination port number → 23

PRACTICAL-10

AIM: Capture and Study TCP and UDP Packets using relevant software.

Absolutely! Let’s go step by step on how to capture and study TCP and UDP packets using tools like Wireshark, which is the most common and user-friendly option for labs.

## 1. Tools Required

- Wireshark (GUI-based network packet analyzer; free, cross-platform)

- Optional: tcpdump (command-line tool, Linux/macOS)

- Applications to generate traffic:

  - TCP: Web browsers (HTTP/HTTPS), Telnet, SSH

  - UDP: Ping (ICMP isn’t UDP, but use DNS queries, video streaming, DHCP, or custom UDP apps)

## 2. Capturing Packets in Wireshark

- Open Wireshark and select the active network interface (Ethernet/Wi-Fi).

- Set a capture filter to limit captured traffic (optional):

  - For TCP only: tcp

  - For UDP only: udp

  - For both: leave blank or use tcp or udp

- Start capture.

## 3. Generating Traffic

- TCP Traffic Examples:

  - Open a browser and visit a website (http://example.com).

  - SSH/Telnet to a device on your network.

- UDP Traffic Examples:

  - DNS query: nslookup www.google.com

  - Video streaming (YouTube, Netflix, etc.)

  - Custom applications sending UDP packets (like online games).

## 4. Studying TCP Packets

TCP Header Fields to Observe in Wireshark:

- Source Port / Destination Port

- Sequence Number / Acknowledgment Number

- Header Length / Flags (SYN, ACK, FIN, RST, PSH, URG)

- Window Size

- Checksum

Steps:

- Click on a TCP packet in Wireshark.

- Expand Transmission Control Protocol section in the packet details.

- Observe:

  - Connection establishment: SYN, SYN+ACK, ACK (3-way handshake)

  - Data transfer: sequence numbers and acknowledgment numbers

  - Connection termination: FIN, FIN+ACK

## 5. Studying UDP Packets

UDP Header Fields to Observe:

- Source Port / Destination Port

- Length (header + data)

- Checksum

Steps:

- Click on a UDP packet.

- Expand User Datagram Protocol section.

- Observe:

  - Source/destination ports

  - Payload size

  - Checksum validity

Tip: UDP is connectionless — there’s no handshake, and sequence numbers are absent.

## 6. Analysis Tips

- Compare TCP vs UDP behavior:

  - TCP = reliable, connection-oriented, uses handshakes, acknowledgments, retransmissions.

  - UDP = unreliable, connectionless, faster, no handshake.

- Identify applications from port numbers:

  - TCP 80 → HTTP, TCP 443 → HTTPS, TCP 22 → SSH

  - UDP 53 → DNS, UDP 123 → NTP, UDP 67/68 → DHCP

- Use Follow TCP Stream / Follow UDP Stream in Wireshark to see the full conversation between endpoints.

## 7. Optional: Command-Line Capture with tcpdump

# Capture TCP packets

sudo tcpdump -i eth0 tcp

# Capture UDP packets

sudo tcpdump -i eth0 udp

# Capture both TCP and UDP and save to file

sudo tcpdump -i eth0 tcp or udp -w capture.pcap

You can then open capture. pcap in Wire shark for detailed analysis.

PRACTICAL-11

## AIM: Configure Dynamic Host Configuration Protocol using relevant software.

## 1. Topology

We'll create a simple network with:

PC1 --- Switch --- Router (DHCP Server)

PC2 --- Switch

- Router (R1) will act as the DHCP server.

- Switch connects PCs and Router.

- LAN network: 192.168.1.0/24

## 2. IP Addressing Plan

- Router (R1) G0/0: 192.168.1.1/24

- PC1, PC2: Will receive IP addresses dynamically from the router.

- Default Gateway for PCs: 192.168.1.1

## 3. Configuring DHCP on a Cisco Router

### Step 1: Configure Router Interface

enable

conf t

interface g0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

### Step 2: Create DHCP Pool

ip dhcp pool LAN1

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1      # Gateway for clients

dns-server 8.8.8.8              # Optional DNS

domain-name example.com          # Optional

### Step 3: Exclude Addresses from DHCP Pool

ip dhcp excluded-address 192.168.1.1 192.168.1.10

- This prevents the router or critical devices from being assigned IPs dynamically.

## 4. Configuring PCs to Use DHCP

- On PC1 and PC2 (in Packet Tracer):

  - Click the PC → Desktop → IP Configuration.

  - Select DHCP.

- The PC should automatically receive an IP like 192.168.1.11, subnet mask 255.255.255.0, default gateway 192.168.1.1, and DNS 8.8.8.8.

## 5. Verification

### On PCs:

ipconfig /all   # Windows

ifconfig       # Linux

- Check IP address, subnet mask, default gateway, and DNS.

### On Router:

show ip dhcp binding      # Shows assigned IP addresses

show running-config       # Check DHCP pool configuration

ping 192.168.1.X          # Test connectivity between PCs

## 6. Optional: DHCP Relay (if DHCP server is on a different subnet)

If clients and DHCP server are on different subnets:

interface g0/1

ip address 192.168.2.1 255.255.255.0

ip helper-address 192.168.1.1   # IP of DHCP server

This forwards DHCP requests from other subnets to the DHCP server.

PRACTICAL-12

AIM:  a) Configure Domain Name Server (DNS) using relevant software.

b) Configure Web Server using relevant software.

### Step 1: Install DNS Server Role

- Open Server Manager → Manage → Add Roles and Features.

- Choose DNS Server → click Next → Install.

### Step 2: Create a Forward Lookup Zone

- Open DNS Manager (dnsmgmt.msc).

- Right-click on Forward Lookup Zones → New Zone.

- Choose Primary Zone → Next.

- Enter your domain name, e.g., example.com.

- Allow it to create a new zone file automatically.

### Step 3: Add Host (A) Records

- Right-click the new zone → New Host (A or AAAA).

- Name: www

- IP address: 192.168.1.10

- Click Add Host.

### Step 4: Configure Reverse Lookup Zone (Optional)

- Right-click Reverse Lookup Zones → New Zone.

- Enter network ID (e.g., 192.168.1).

- Add PTR record mapping IP → domain name.

### Step 5: Test DNS

On any client PC:

nslookup example.com <DNS_Server_IP>

b) Configure Web Server using relevant software.

### Step 1: Install Apache Web Server

Open your terminal and run:

sudo apt update

sudo apt install apache2 -y

Once installed, check its status:

sudo systemctl status apache2

If it’s not running:

sudo systemctl start apache2

sudo systemctl enable apache2

### Step 2: Test Apache Installation

Open a web browser and enter:

http://localhost

PRACTICAL-13

AIM-Configure File Transfer Protocol (FTP) using relevant software.

### Step 1: Download and Install FileZilla Server

- Download from https://filezilla-project.org/.

- Install and launch FileZilla Server Interface.

### Step 2: Configure the Server

- When prompted, connect to the local server (default port 14147).

- Go to Edit → Settings.

- Under General settings, ensure the Listen on these ports is set to 21.

### Step 3: Add an FTP User

- Go to Edit → Users.

- Click Add → enter username (e.g., ftpuser).

- Under Shared folders, click Add and choose a folder to share (e.g., C:\FTPData).

- Set permissions (check Read, Write, Delete, Append if needed).

- Click OK to save.

### Step 4: Test the FTP Server

Open Command Prompt or a browser and type:

ftp://localhost

Or connect from another computer:

ftp://<Server-IP>

PRACTICAL-14

AIM-Configure Mail Server Using relevant software.

### Step 1: Download and Install hMailServer

- Download from https://www.hmailserver.com

- Install and open hMailServer Administrator.

### Step 2: Connect to the Local Server

- Open hMailServer Administrator.

- Choose localhost and enter the password set during installation.

### Step 3: Add a Domain

- Go to Domains → Add.

- Enter your domain name (e.g., example.com).

- Click Save.

### Step 4: Add Email Accounts

- Select your domain → Accounts → Add.

- Enter:

  - Address: user1@example.com

  - Password: 12345

- Click Save.

Repeat for other users.

### Step 5: Configure SMTP, POP3, and IMAP

- Go to Settings → Protocols and ensure all three (SMTP, POP3, IMAP) are enabled.

- Set the local hostname under Settings → Protocols → SMTP → Delivery of e-mail.