Faculty of Engineering (Diploma)

Semester –5TH CE

Subject Name- COMPUTER ORGANIZATION AND ARCHITECTURE

Subject Code:- DE103504

Faculty Name –Khushbu Goswami

# Unit :V

# Input-Output Organization

## Explain I/O interface

    - It consists of two data registers called ports, a control register, a status register, bus buffers, and timing and control circuit.

    - The interface communicates with the CPU through the data bus.

    - The chip select and register select inputs determine the address assigned to the interface.

    - The I/O read and writes are two control lines that specify an input or output,respectively.

    - The four registers communicate directly with the I/O device attached to the interface.

    - A command is passed to the I/O device by sending a word to the appropriate interface register.

    - The control register receives control information from the CPU.

| C S | RS 1 | RS 0 | Register Selected |
| --- | --- | --- | --- |
| 0 | X | X | None: data bus in high impedance |
| 1 | 0 | 0 | Port A register |
| 1 | 0 | 1 | Port B register |
| 1 | 1 | 0 | Control register |
| 1 | 1 | 1 | Status register |

    - For example, port A may be defined as an input port and port B as an output port.

    - The interface registers communicate with the CPU through the bidirectional data bus.

    - The address bus selects the interface unit through the chip select and the two registerselect inputs.

    - The two register select inputs RS1 and RS0 are usually connected to the two leastsignificant lines of the address bus.

    - These two inputs select one of the four registers in the interface as specified in the table accompanying the diagram.

    - The content of the selected register is transfer into the CPU via the data bus when the I/O read signal is

enables.

    - The CPU transfers binary information into the selected register via the data bus when the I/O write input is enabled.

## Differentiate various Modes of Data Transfer with I/O

    - Data transfer to and from peripherals may be handled in one of three possible modes.

      - Programmed I/O

      - Interrupt initiated I/O

      - Direct Memory Access(DMA)

## Programmed I/O

    - The I/O device does not have direct access to memory.

    - A transfer from I/O device to memory require the execution of several instruction by CPU(device to CPU and CPU to memory, no of words transferred)

    - When a byte of data is available, the device places it in the I/O bus and enables its data valid line.

    - The interface accepts the byte into its data register and enables the data accepted line.

    - The interface sets a bit in the status register that we will refer to as an F or "flag" bit.

    - The device can now disables the data valid line.

    - Then reading the status register into a CPU register and check the value of the flag bit .

    - If the flag is equal to 1, the CPU reads the data from the data register.

    - The flag bit is then cleared to 0 by CPU or Interface(depends o IC)

    - Then Interface disable the data accepted line and the device can transfer the next data byte.

## Interrupt Initiated I/O.

    - In programmed initiated, CPU stays in a program loop until the I/O unit indicates that it is ready for data transfer.

    - It keeps the processor busy without need.

    - It can be avoided by using an interrupt facility .

    - When the data are available from devices, interface issues an interrupt request signal.

    - In the meantime CPU can proceed to execute another program.

    - The interface meanwhile keeps monitoring the device.

    - When the interface determines that the device is ready for data transfer, it generates an interrupt request to the computer.

    - Upon detecting the external interrupt signal, CPU stops the task, branches to the service program to process I/O and then return to the task it was originally performing.

## DMA(Direct Memory Access)

    - The transfer of data between a fast storage device such as magnetic disk and memory is often limited by the speed of the CPU.

    - Removing the CPU from the path and peripheral device manage the buses directly will improve the speed of transfer.

    - This transfer technique is called DMA.

    - A DMA controller takes over the buses to manage the transfer directly between the I/O devices and memory.

    - BR is used by DMA Controller to request CPU to relieve control of buses.

    - BR is active, then CPU place address bus, data bus, read and write line into high impedance state.

- The CPU activate the BG to inform DMA that buses are in high impedance state.

- DMA takes the control of buses and conduct memory transfer.

- When the DMA terminates the transfer, it disable the bus request line(BR).

- CPU disable the BG.

    - The register in the DMA are selected by the DS (DMA select) and RS (register select) inputs.

    - The RD (read) and WR (write) inputs are bidirectional.

    - When the BG is 0, the CPU can communicate with the DMA registers through the data bus to read from or write to the DMA registers.

    - When BG= 1, the CPU relieve the buses and the DMA can communicate directly with the memory

    - The word count register holds the number of words to be transferred.

## Describe Input-Output Processor (IOP)

    - Instead of having each Interface communicate with CPU, computer may have one or more external processor.

    - Each processor communicates directly with I/O devices.

    - IOP may be classified as a processor with direct memory access capability that communicates with I/O devices.

    - IOP provides a path for transfer of data between various peripheral devices and the memory unit.

    - IOP operates independent of the CPU.

## Describe CPU-IOP communication

    - CPU send an instruction to test the IOP path.

    - The IOP responds by inserting a status word in memory for CPU.

    - Status word indicate the condition of the IOP and I/O device(IOP overload condition, device busy, device ready)

    - The CPU check the status word in memory to decide what to do.

    - If status is OK, send start I/O instruction to IOP.

    - Now CPU is continue with another program while IOP is busy with I/O program.

    - When IOP terminates the execution, it send interrupt request to CPU.

    - CPU send request for IOP Status.IOP send status report. It indicates the transfer has been completed or if any error occurs during the transfer.CPU check status word for correct transfer.